oo_v1 Reverse-GNSS Simulation

Technical Documentation

An object-oriented MATLAB simulation of ground-to-space GNSS-like navigation for GEO spacecraft: EKF state estimation, multi-antenna attitude, two-way time transfer, and inter-satellite-link swarms.

Cranfield University - Individual Research Project (IRP)

Repository: IRP_Simulation/oo_v1  -  MATLAB R2025b

August 2026

Contents

# 1. Introduction and Purpose

oo_v1 is a clean, object-oriented MATLAB (R2025b) simulation of a reverse-GNSS navigation architecture: several ground towers transmit GNSS-like ranging signals upward to one or more space assets, and each asset estimates its own position, velocity, attitude, angular rate and receiver-clock state with an Extended Kalman Filter (EKF). The primary study case is a geostationary (GEO) spacecraft; MEO and LEO orbit classes are also supported.

The simulation exists to answer quantitative questions about this architecture:

- What orbit and time accuracy can a GEO asset reach from a small network of ground uplink towers, with honest error models for clocks, atmosphere, hardware and geometry?

- Which augmentations move the needle: multi-antenna carrier-phase attitude, two-way satellite time and frequency transfer (TWSTFT), inter-satellite links (ISL), carrier phase, and multi-satellite swarms?

- Where are the physical observability walls (for example the single-GEO radial/clock degeneracy, and formation-rotation blindness of pure ISL ranging), and what breaks them?

The validated headline case is a single GEO asset with a 4-antenna cross (GNSS lever-arm attitude on). The nominal default configuration uses one antenna with a star-tracker/gyro attitude solution. An optional ISL swarm of secondary satellites and an optional TWSTFT observable can aid the primary; a federated multi-asset estimation mode runs one EKF per asset plus a dedicated relative layer.

## 1.1 Design principles

- One entry point. Every run goes through run_oo_v1.m; there are no standalone science scripts. Reusable logic lives in classes.

- One configuration file. All run configuration lives in config/masterConfig.m; the runner adds no physics toggles of its own. Scenario JSON files overlay the master config, never replace it.

- Gated features, honest defaults. New physics and estimator features ship behind master enables that default off (or to the frozen behaviour), so the golden regression stays byte-identical unless a change is intentional.

- Truth/estimation separation. Truth models and estimator models are built separately and may be deliberately mismatched; a model-family guard prevents accidental identity.

- Self-describing runs. Every run writes its fully resolved configuration plus a literal-vs-resolved override list into a MATLAB-free .out log, so a result can be interpreted without re-opening MATLAB.

# 2. System Concept

## 2.1 Reverse-GNSS topology

In clock and estimation topology, reverse GNSS is identical to forward GPS: many transmitters (towers, each with its own clock) and one receiver per asset (one common receiver clock). The pseudorange for tower i is:

z_i = rho_i + b_rx - b_tower_i + trop_i + iono_i + hardware + multipath + noise

with the receiver clock common to every row (+1) and each tower clock entering per row (-1). The "reverse" is purely geometric - transmitters sit on the rotating Earth and the receiver is in space - which changes elevation geometry, the atmospheric path direction, Sagnac and light-time bookkeeping, and observability.

## 2.2 Measurement geometry and sign conventions

The antenna phase centre is offset from the centre of mass through the body attitude, which is what makes attitude observable from ranging at all:

antenna phase centre : r_ant = r_cm + C_ecef_body(euler) * leverArm_body

truth pseudorange    : z_i = ||r_ant_true - r_tower_i|| + b_rx - b_tower_i

+ trop + iono(+code) + hardware + multipath + noise

model pseudorange    : h_i = ||r_ant_est  - r_tower_i|| + b_rx_est - b_tower_model_i

+ trop_model + iono_model(+code)

Verified sign conventions: line-of-sight Jacobian +u (tower-to-spacecraft unit vector); receiver clock +1; tower clock -1; ionosphere + for code and - for carrier; troposphere + for both. Attitude is observable only through the lever arm and (with two or more antennas) carrier phase; it is weak from code alone.

## 2.3 Known observability walls

- Radial-clock degeneracy (single GEO, one-way uplink). From a compact regional tower network, a radial position shift and a receiver-clock shift are nearly indistinguishable; the correlation reaches -1.000. This persists across uplink frequencies and also appears at MEO. It is broken by two-way time transfer (which pins the clock independently), by a wider tower network, or by swarm aiding.

- Formation rotation is invisible to ISL ranging. A rigid rotation of the whole formation preserves every inter-satellite range exactly, so no amount of crosslink ranging precision can observe it. Ground-referenced observables (differenced carrier) or long arcs with orbital dynamics (anisotropic Clohessy-Wiltshire) are required; the governing variable is how far the formation turns during the arc.

- Common-mode absorption. Common-mode delays (atmosphere, clock offsets) are partly absorbed by the receiver-clock state. This is physically correct - atmosphere modelling improves innovation RMS more than position accuracy.

# 3. Running the Simulation

## 3.1 Quick start

cd oo_v1

run_oo_v1                          % masterConfig overlaid by default.json

run_oo_v1('realism.json')          % masterConfig overlaid by a scenario JSON

out = run_oo_v1(...)               % returns the ReportRunner output struct

run_oo_v1 resolves config/masterConfig.m through exactly one JSON overlay from config/scenarios/, creates a self-describing per-run output folder, runs the pipeline (single-asset or federated swarm, chosen inside), and writes the report artefacts. Headless execution uses matlab -batch (the MATLAB MCP path is -nojvm and times out on long runs).

## 3.2 Main run knobs (top of masterConfig.m)

| Knob | Meaning | Default |
| --- | --- | --- |
| cfg.scenario.nTowers | Ground transmitters (12 real sites defined; 5 = the frozen network) | 5 |
| cfg.scenario.nSpaceAssets | 1 = ground-only; >1 = helix ISL swarm aiding the primary | 1 |
| cfg.scenario.nReceivers | 1 = star-tracker/gyro attitude; >=2 = 4-antenna cross, GNSS attitude on | 1 |
| cfg.scenario.orbitClass | Orbit class: 'GEO' ¦ 'MEO' ¦ 'LEO' | 'GEO' |
| cfg.simulation.duration_s | Run length in seconds | 14400 (4 h) |
| cfg.simulation.dt_s | Simulation and measurement time step | 1 s |
| cfg.asset.clockType | Oscillator class: 'CESIUM1' ¦ 'OCXO' ¦ 'RUBIDIUM' ¦ 'TCXO' | 'CESIUM1' |
| cfg.clock.templateSource | Clock realism: 'legacy' (idealised) ¦ 'jowTable2p1' (literature-anchored) | 'legacy' |
| cfg.measurements.twstft.enable | Two-way time-transfer observable on/off | false |
| cfg.physics.relativity.clock.enable | Gated relativistic clock-rate offset (truth side) | false |
| cfg.atmosphere.realistic | Realistic troposphere/ionosphere overlay | true |
| cfg.atmosphere.sharedAcrossFormation.enable | One per-tower air column for the whole swarm | false |
| cfg.signals.enabledMask | Single frequency control; [true,true] = L1+L2 | [true,true] |

## 3.3 Companion tools

| Script | Purpose |
| --- | --- |
| run_ladder.m / run_error_ladder.m | Topology and error-budget ladders; every rung of one run grouped under a single folder with a combined summary. The 14-rung error ladder walks from idealised to realism-graded configurations. |
| run_oo_v1_battery.m / run_oo_v1_freqbattery.m | Batch batteries over scenario grids (e.g. the scene_* JSON grid, frequency sweeps). |
| run_oo_v1_analysis.m | Post-run analysis over stored run databases. |
| plot_mat_report.m | Interactive viewer for a saved run .mat (re-plot without re-running). |
| regenerate_report_from_mat.m | Rebuild the report artefacts from a stored .mat. |
| tests/run_all_tests.m | The full unit-test suite (run with tests/ on the path). |
| tests/regression/run_oo_v1_regression.m | The frozen golden regression gate (see Section 10). |

# 4. Architecture

## 4.1 Repository layout

| Path | Contents |
| --- | --- |
| run_oo_v1.m | THE entry point: resolve config, run, report. Thin by design. |
| config/ | masterConfig.m (the single config file, with inlined base defaults), resolveSimulationConfig.m (applies exactly one JSON overlay), scenarios/ (~130 scenario JSONs), internal helpers. |
| +revgnss/ | The main package (~220 classes): orchestration, scenario/config factories, observable builders, swarm and distributed estimation, diagnostics, report builders. |
| +filter/ | ReverseGNSSEKF.m (the EKF) and EkfDynamicsPredictor.m (propagation, F and Q). |
| +models/ | Physics and measurement models in sub-packages: +clocks, +measurements, +errors, +atmosphere, +corrections, +frames, +orbit, +noise, +sensors. |
| +data/ | SimulationDataStore.m - flat per-epoch diagnostics store (position/clock/attitude/NIS/NEES). |
| tests/ | Unit tests (test_*.m, several hundred), golden references, regression gate, report tests. |
| docs/ | Design documents, audits, execution plans, validation manual. |
| output/ | Per-run report folders (Section 9) plus output/latest/ convenience pointers. |

## 4.2 Execution pipeline

A run flows through a fixed pipeline:

- masterConfig.m builds the literal configuration: structural defaults, user toggles, scenario assembly, contract check.

- resolveSimulationConfig overlays exactly one scenario JSON (default.json if none is named) and records which keys the scenario owns.

- ConfigFactory.finalizeConfig resolves the literal config into the operative one - rebuilds lever arms, clocks, the atmosphere profile, frequency masks, the time vector and thresholds. The literal masterConfig is deliberately not the whole story; the .out log records what changed.

- ReportRunner.runSingle drives the run: simulate, post-process, and write the PDF/MAT/.out/.tex artefacts. It selects the single-asset or federated-swarm path.

- ScenarioFactory instantiates the asset(s), towers, EKF, measurement models and error chains from the resolved config.

- ReverseGNSSSimulation is the epoch loop orchestrator: truth propagation, measurement synthesis, EKF predict, EKF update, diagnostics capture into SimulationDataStore.

## 4.3 Key classes

| Class | Role |
| --- | --- |
| revgnss.ReverseGNSSSimulation | Top-level orchestrator (truth -> measure -> predict -> update); owns assets, towers, models, EKF, data store, carrier track managers. |
| revgnss.ConfigFactory | finalizeConfig, presets, clock templates, atmosphere profile resolution. |
| revgnss.ScenarioFactory | Builds the scene from cfg (assets, towers, EKF, measurement and error models). |
| revgnss.ReportRunner | Single-run driver: simulate + optional PDF/MAT/.out/.tex report; federated swarm path. |
| revgnss.ClockExactReportBuilder | The production LaTeX report builder. |
| filter.ReverseGNSSEKF | 14+-state EKF: Joseph-form update, quaternion attitude error-state with reset. |
| filter.EkfDynamicsPredictor | State propagation, state-transition matrix F and process noise Q. |
| models.clocks.ClockModel | IEEE-1139 power-law oscillator truth (WPM/FPM/WFM/FFM/RWFM). |
| models.measurements.MeasurementModel | Code / carrier / Doppler measurement and Jacobian builders. |
| models.errors.ErrorChain / EnvironmentModel | Per-asset error chain: atmosphere, hardware, multipath, noise draws. |
| models.noise.RngRegistry | Threefry counter-based RNG, identity-keyed streams for reproducibility. |
| revgnss.SpaceAsset / GroundTower / SwarmFormation | Physical actors: estimated asset(s), transmitter sites, helix formation geometry. |
| data.SimulationDataStore | Flat per-epoch diagnostics store written by the loop, read by reports. |

# 5. Configuration System

masterConfig.m is the single configuration file and reads top to bottom: the structural defaults (an inlined subfunction, formerly baseConfig.m), then user toggles grouped by topic, then the scenario assembly that runs after the toggles (and may override a few of them), then a contract check. masterConfig('baseOnly') returns just the structural defaults; derived, test and frozen-golden configurations are all built on those same defaults.

Scenario JSONs in config/scenarios/ are overlays, applied by resolveSimulationConfig - exactly one per run. They range from the default and realism profiles, through systematic grids (scene_G#S#R#_ts#_TW#_*), error-budget ablations (budget_a*), Ka-band studies (ka_c*), numbered experiment configurations (run1 ... run22), to golden regression pins (golden_*) and the ground-orientation research scenarios (ground_orientation_*).

Configuration-resolution trap: finalizeConfig can override literal masterConfig values (for example folding a standalone Sagnac correction into iterative light-time, or resolving codeMode). This is why every run stores the fully resolved config and the literal-vs-resolved override list in its .out file - always check the resolved values, not just the literal file, when interpreting a run.

Realism is graded rather than binary: gated realism-grade and honest-covariance configurations exist alongside the optimistic legacy defaults. The default configuration is deliberately the frozen (optimistic) baseline; realism overlays degrade it honestly. The clock is the largest single realism lever (cfg.clock.templateSource = 'jowTable2p1').

# 6. State Vector and Estimator

## 6.1 State vector

The base state dimension is 14:

x = [ r(3)  v(3)  euler(3)  omega(3)  b_rx  bdot_rx ]

It is augmented per run with:

- Carrier float ambiguities - one state per (tower x receiver x signal) arc, managed by an ambiguity state registry with cycle-slip detection and arc bookkeeping.

- Per-tower zenith wet delay (ZWD) and per-tower slant ionosphere states, when the corresponding estimation options are enabled.

- Two clock states per tower when estimateTowerClocks is enabled.

- A gated solar-radiation-pressure scale state (default off).

In joint multi-asset mode the per-asset blocks are stacked into one filter through an asset-symmetric state-block resolver (AssetStateBlock), so every asset resolves its indices the same way.

## 6.2 Filter mechanics

- Update: Joseph-form covariance update for numerical symmetry/stability.

- Attitude: quaternion attitude with an error-state formulation and post-update reset; Euler angles are the reported representation.

- Jacobians: measurement Jacobians are built analytically for the core observables; the finite-difference measurement-Jacobian path is the dominant compute cost of a run (about a third of runtime), ahead of state-transition-matrix work.

- Clock process noise: the Brown-Hwang two-state (bias, drift) Q derived from the dominant white-FM and random-walk-FM terms of the oscillator model.

- Dynamics: J2 orbit propagation in the truth and the filter model (same family, deliberately controlled by the truth-estimation separation guard).

## 6.3 Convergence behaviour

Pseudorange from N towers primarily observes position (3 degrees of freedom) and receiver clock bias (1). With few measurements and many states the EKF is per-epoch underdetermined but converges recursively. Position and clock converge first, typically within 600-1800 s; attitude converges slowly (weak observability through the lever arm). Once converged, NIS sits near the number of visible measurements; the shipped filter is conservative by design and is expected to read below the chi-square consistency band (under-confident, honest).

# 7. Observables and Measurement Models

| Observable | Description | Default |
| --- | --- | --- |
| Code pseudorange | Ground-to-space one-way uplink code, per tower / receiver / signal; L1+L2 with iono-free combination available. | on |
| Carrier phase | Same links with float ambiguity states; cycle-slip detection; iono-free carrier; wide-lane / narrow-lane diagnostics. No integer fixing by default. | on |
| Doppler / range-rate | One-way Doppler, aids velocity and clock drift. | on |
| TWSTFT (ground-space) | Two-way satellite time and frequency transfer; pins the receiver clock and breaks the radial-clock degeneracy; enables the sub-100 ps regime. | off |
| ISL one-way code | One-way inter-satellite code ranging (swarm aiding of the primary). | off |
| ISL coherent two-way code | Coherent two-way inter-satellite code ranging with hardware model and light-time closure (four-event oracle). | off |
| ISL four-timestamp time transfer | TWSTFT-like satellite-to-satellite clock difference from four timestamps; sign convention b_remote - b_reference. | off |
| ISL carrier | Inter-satellite carrier phase with its own arc/slip manager; float ambiguities (undifferenced ISL carrier is measured to be inert - the float ambiguity absorbs its contribution). | off |
| Ground-relay time-transfer sessions | Time transfer relayed through swarm members to the primary (session-based observable builders for the distributed path). | off |
| Star tracker + gyro | Attitude sensor suite (IMUModel, StarTracker/Gyroscope measurement models); the nominal attitude solution for 1-antenna runs. | on (R1) |
| Multi-antenna GNSS attitude | Lever-arm attitude from the 4-antenna cross via carrier phase; the explicit attitude-experiment configuration. | on (R4) |

Ambiguity resolution. Integer ambiguity machinery exists (LAMBDA-class search in +revgnss/+integer, a baseline resolver for the ground attitude problem, and a decorrelated-bootstrap partial fixer on the research branch) but the shipped default runs carrier as float ambiguities. Measured behaviour: ground-attitude baseline ambiguity resolution fixes reliably (success rate 1.000); ISL double-difference ambiguity resolution correctly refuses at current precision - the observable formulation is right, crosslink precision is the blocker.

# 8. Truth and Error Models

## 8.1 Orbit and frames

- J2 orbit propagator with configurable perturbations; DE440 ephemeris support for third-body terms; GEO / MEO / LEO scenario classes.

- Frame and time utilities with a simplified constant Earth-rotation rate (no IERS/EOP-grade frames - a stated limitation); solid-Earth tide model for tower displacement.

- Light time solved iteratively (one-way, 2 iterations to 1e-12 s tolerance); Sagnac handled inside the iterative light-time path; Shapiro delay enabled by default.

## 8.2 Clocks

ClockModel implements the IEEE-1139 power-law PSD S_y(f) = h2 f^2 + h1 f + h0 + h-1/f + h-2/f^2. White-FM and random-walk-FM propagate directly in the time domain; WPM/FPM/FFM are FFT-synthesised over the run. Oscillator classes: CESIUM1, OCXO, RUBIDIUM, TCXO. Clock realism is one string: 'legacy' is the idealised set (about seven orders of magnitude too quiet in ADEV terms), 'jowTable2p1' is the literature-anchored realistic set. A known limit: for a caesium receiver the drift wander sits far below the Doppler resolution, so the drift +-3-sigma envelope is observability-limited - a measured property, not a filter bug. The relativistic clock-rate offset (gravitational + special-relativistic, about 5.4e-10 fractional for GEO vs ground) is a gated truth-side term; for a circular orbit the constant offset is fully absorbed by the estimated drift state.

## 8.3 Atmosphere

Troposphere and ionosphere have simple mapped nominal models (independent truth and correction values) plus a realistic overlay: Niell mapping / Saastamoinen-class troposphere and Klobuchar-class ionosphere, higher-order ionosphere terms, stochastic processes and scintillation. Each asset owns an EnvironmentModel rooted at its own seed, so by default two swarm members draw independent atmosphere realisations. Physically a formation 2 km across at GEO sees one tower through ray paths diverging by only ~11 arcsec - the delay is common-mode - so cfg.atmosphere.sharedAcrossFormation.enable re-roots the atmosphere streams at a formation-wide seed. Turn it on for any between-satellite differenced ground observable; it shares the atmosphere only (receiver noise, clocks, multipath and hardware stay per-asset).

## 8.4 Hardware, noise and reproducibility

- Error chain per asset: hardware delays (signal-dependent delays default to zero; code DCB is global per signal - no calibrated per-link product), multipath, receiver noise, antenna PCO/PCV.

- RngRegistry provides threefry counter-based, identity-keyed random streams: every noise consumer draws from a named stream derived from the master seed, so runs are exactly reproducible and per-effect toggles do not shift unrelated draws.

- Truth and estimator atmospheric/clock parameters are deliberately mismatched (e.g. tropo zenith delay truth 2.45 m vs model 2.30 m) so residual errors are realistic rather than zero.

# 9. Multi-Asset and Swarm Estimation

## 9.1 Modes

| Mode | What runs | Character |
| --- | --- | --- |
| Single asset | One EKF on the primary; ground observables only. | The baseline; metre-to-decimetre class navigation, clock-limited one-way. |
| Joint | One stacked EKF over all assets (asset-symmetric state blocks) with ISL observables as in-filter rows. | Full cross-covariance, but the joint filter dilutes crosslink information across many states. |
| Federated | N independent per-asset EKFs plus a dedicated relative layer (at most 5 neighbours per asset) on ISL/TWSTFT observables. | The pivot architecture: dramatically better shape and relative clock than the joint filter at matched budgets. |
| Distributed / independent fleet | IndependentFleetCoordinator: per-node fusion with a distributed cross-covariance network, pair-exact covariance updates, split covariance-intersection bound, TWSTFT relay sessions and delivery ledgers. | The decentralised path; heavily guarded configuration contract (one observable type at a time, calibrated links, fast mode). |

## 9.2 Formation and relative solution

Multi-asset scenarios fly a helix formation (Clohessy-Wiltshire relative motion) of secondaries around the primary. Swarm aiding scales the primary clock roughly as 1/sqrt(N-1); the configured design point for the aided primary is centimetre-class position and tens-of-picoseconds clock. The relative layer includes SwarmRelativeSolver, a distance-geometry (classical multidimensional scaling) shape solution over the two-way ISL ranges - shape error tracks the effective ISL range error, and with calibrated crosslink hardware it reaches the sub-millimetre class on synthetic budgets. The remaining relative degree of freedom is formation rotation, which pure ranging cannot see (Section 2.3).

## 9.3 Active research: ground-referenced orientation (gated, default off)

The current research branch adds machinery to recover formation orientation from ground-referenced carrier observables. All of it is gated and the golden runs are byte-identical with the gates off:

- GroundDifferencedRotationSolver - between-satellite differenced ground carrier as a rotation observable (validated: recovers an injected rotation exactly in the clean case), with a shape-leakage guard because arc-correlated shape error projects onto rotation.

- JointGeometrySolver - joint arc-constant shape + rotation least squares (3N+3 parameters); separability is governed by the arc turn angle, so short arcs cannot split shape from rotation and long arcs can.

- GroundCarrierAmbiguityResolver + integer.DecorrelatedBootstrap - double-difference ambiguity handling for the ground carrier, with partial (bootstrapped) fixing in a decorrelated basis.

- OrientationCoherenceBudget, ShapeFrameSeparationProbe, GoldenRunFingerprint - budget/diagnostic probes and run fingerprinting for reproducibility of the research scenarios.

# 10. Outputs and Reporting

## 10.1 Run folder layout

output/Report_YYYYMMDD/Report_v###_G#S#R#_TW#/Report_v###_ts#_G#S#R#_TW#.{pdf,mat,out,tex}

v###  = cfg.report.runVersion     ts#  = duration in seconds

G/S/R = towers / space assets / receivers   TW# = TWSTFT on (1) / off (0)

Ladder runs group every rung under one Ladder_{description}/ folder with a combined text summary. output/latest/ holds convenience copies of the newest PDF/MAT per scenario name.

## 10.2 Artefacts

| Artefact | Contents |
| --- | --- |
| .pdf | The production report (LaTeX, clockExact layout, compiled with pdflatex/xelatex; report format version tracked in the document). Error tables, convergence and consistency plots, clock and atmosphere sections, swarm appendix for federated runs. |
| .mat | The SimulationDataStore diagnostics (compact mode by default; sampledFull/full add matrix snapshots). Re-plottable with plot_mat_report.m. |
| .out | MATLAB-free run log: final metrics, output paths, the full resolved config, and the literal-vs-resolved override list. Optional Monte-Carlo consistency appendix. |
| .tex | The LaTeX source of the report. |
| PNG diagnostics | Atmosphere residual diagnostics for single-asset runs (atmosphere_residuals_time.png and friends). |

# 11. Verification and Validation

## 11.1 Frozen golden regression gate

tests/regression/run_oo_v1_regression('smoke')              % 120 s single-antenna

tests/regression/run_oo_v1_regression('full')               % 14400 s single-antenna

tests/regression/run_oo_v1_regression('smoke','headline')   % 4-antenna headline

Two frozen references (single-antenna baseline and 4-antenna headline). The gate re-runs the frozen scenario and fails if any core scientific metric moves beyond floating-point tolerance; non-core diagnostic changes only warn. The gate certifies that refactors never move the scientific numbers, and it is never weakened to make a change pass.

## 11.2 Unit and consistency testing

- Several hundred unit tests under tests/ (run via tests/run_all_tests.m) covering Jacobian signs, observability, config contracts, per-effect toggles, report semantics and the distributed-estimation contracts.

- MonteCarloConsistency runs an ensemble (initial error drawn from P0, measurement and clock-truth seeds varied per draw), pools per-epoch NIS/NEES and band-checks the pooled sums against a two-sided chi-square interval (Bar-Shalom bounds). This is synthetic consistency evidence, not real-world proof, and is off by default.

- Audit classes (ImperfectionAudit, ModelCoverageAudit, AttitudeJacobianAudit, observability diagnostics) assert model coverage and Jacobian correctness in-run.

## 11.3 External cross-validation (Orekit)

An external validation bridge (kept outside the repository) cross-checks the simulation against Orekit. Measured agreement: the simulation EKF against an equivalently configured Orekit KalmanEstimator to ~7e-6 m in state and ~6e-6 in relative covariance; an Orekit batch least-squares given only the simulation pseudoranges recovers the simulated truth to below 1e-7 m from a 50 km initial error; light-time composition and superposition checks are exact; the LAMBDA integer search cross-validates 100/100 against the Orekit implementation. This proves implementation equivalence, not statistical validity: the process-noise model is untested by that comparison (it was zeroed to make the filters comparable), so covariance honesty rests on the consistency and audit machinery above.

# 12. Limitations and Honest Claims

- One-way uplink is clock- and geometry-limited; the two-way (TWSTFT) and ISL-swarm paths reach the sub-metre / sub-100 ps regime, a one-way link alone does not.

- No integer ambiguity fixing in the shipped default - carrier runs as float ambiguities (integer machinery exists behind gates and for the attitude baseline).

- No calibrated hardware bias / DCB / phase-bias products; no ANTEX / IONEX / SP3 / CLK / RINEX ingestion.

- No IERS/EOP-grade reference frames; simplified constant Earth-rotation rate.

- Signal-dependent hardware delays default to zero; configured code DCB is global per signal; the Doppler ionosphere-rate term is not modelled; the pseudorange/Doppler shared tower-clock cross-covariance is ignored (block-diagonal R).

- Default configuration is the optimistic frozen baseline; realism grades (clock template, honest covariance) must be switched on deliberately and degrade the results honestly.

- Filter consistency: NEES well above 1 in some stressed configurations reflects a real observability wall, not a filter defect; the shipped filter is deliberately conservative and reads below the chi-square band.

# 13. Glossary

| Term | Meaning |
| --- | --- |
| Reverse GNSS | Ground transmitters, space receiver: GNSS-like uplink navigation. |
| EKF | Extended Kalman Filter. |
| GEO / MEO / LEO | Geostationary / medium / low Earth orbit. |
| ISL | Inter-satellite link (code, carrier, time-transfer observables between swarm members). |
| TWSTFT | Two-way satellite time and frequency transfer. |
| NIS / NEES | Normalised innovation squared / normalised estimation error squared (chi-square consistency statistics). |
| ZWD | Zenith wet delay (troposphere state per tower). |
| DCB | Differential code bias. |
| PCO / PCV | Antenna phase-centre offset / variation. |
| LAMBDA | Least-squares ambiguity decorrelation adjustment (integer ambiguity search). |
| ADEV | Allan deviation (oscillator stability measure). |
| CW | Clohessy-Wiltshire relative orbital dynamics (formation flight). |
| MDS | Multidimensional scaling (distance-geometry shape solution). |
| Joseph form | Numerically symmetric covariance update form of the Kalman filter. |
| Golden gate | Frozen-reference regression test that scientific outputs are byte-identical under refactors. |

Document generated from the repository state on branch feature/ground-orientation-exec, August 2026. The authoritative quick reference is README_oo_v1.md; design history and audits live under docs/.